| FLOOR PLAN |  |  |  |  |
| --- | --- | --- | --- | --- |
| NAME: |  |  |  |  |
| Quality & Completeness of… |  |  |  |  |
| Walls | - Exterior 6" - Interior 4" - Halls and stairways minimum 3' wide |  |  | /2 /2 /2 |
| Windows & Doors | - Doors open INTO the building or room - At least 4" of wall on each side of the door (door post) - Schedule located on Floor Plan |  |  | /2 /2 /2 |
| Closets - doors, shelves and rods | - Clothes/coat 2'6" deep - Closet per bedroom (min 6 sq.ft.) - Closet near the entrance (min 6 sq.ft.) - A linen closet in or near the bathroom(s) – full width shelf but no rod, 2' deep |  |  | /2 /2 /2 /2 |
| Fireplace | - Exists |  |  | /1 |
| Bathroom | - Counter 18" - 24" wide - Sink, tub, and toilet |  |  | /2 /2 |
| Kitchen | - Counter 24" - Upper cupboards 12" - Appliances |  |  | /2 /2 /2 |
| Roof line | - Drawn in a hidden or phantom line |  |  | /2 |
| Dimensions - outside | - 1 dimension line - centre of windows/doors and to outside of ext. walls - 2nd dimension line - centre of interior walls and to outside of ext. walls - 3rd dimension line - outside of ext. walls - 4th dimension line - overall size (outside to outside of furthest ext. walls) NOTE: All dimensions read from bottom and right side NOTE: Do not show the property itself – only the floor plan |  |  | /2 /2 /2 /2 |
| Dimensions - Interior | - Horizontal and vertical dimension line(s) to pick up any interior walls not dimensioned outside |  |  | /2 |
| Electrical | - Lights for each room and hallways - Exterior lights - Switches for lights and attached with double curved (use spline) centrelines - 3 way switches for hallways and stairways (top and bottom) as appropriate - Outlets - 30" from corner of room and every 12' and every 6' on counters - 220 outlets for stove/oven and dryer (electric hot water is 220, gas hot water is 120) - GFI outlet for bathroom and GFI/WP outlet for front and back exterior |  |  | /2 /2 /1 /1 /2 /1 /2 |
| Utilities | - Furnace & Hot Water Tank |  |  | /2 |
| Lettering | - All labeled in all capitals |  |  | /2 |
| Subtotal |  |  |  | /60 |
| Complexity of… - Basic rectangular house 5 - Complex elements, including “kickouts” and/or extra features 10 - Complex elements AND more floors than required 15 |  |  |  | /10 |
| TOTAL |  |  | /70 |  |
| ELEVATIONS |  |  |  |  |
| NAME: |  |  |  |  |
| Quality & Completeness of… |  |  |  |  |
| Walls/Doors/Windows | - Walls 8' high - Accurate to the floor plan (they line up) - Doors 6' - 8' high and 3' wide - Top of window matches top of door height | /2 /2 /2 /2 |  |  |
| Chimney | - Extends 3' above roof where it exits the roof | /2 |  |  |
| Roof | - Standard roof pitch (3/12, 4/12, 5/12, 6/12, etc.) - Roof Pitch indicated on each view - Same height from one elevation to the next - Overhang 6" to 2' - comes from top 8' of wall and 6" up then out - Accurate to floor plan - Roof Style (Gable end = 5, Hip end = 10) | /2 /2 /4 /2 /2 /10 |  |  |
| Dimensions | - Ground to top of the eaves (top of the overhang) - Ground to all roof peaks - Ground to chimney - On all views | /4 /4 /4 /4 |  |  |
| Detail & Labeling | - Window and door trim drawn - Hatch patterns (Stucco, siding, shingles, bricks, etc.) - Materials labeled (use proper names - stucco, siding, shingles, bricks) - Your name, title of drawing, block, scale, etc. | /2 /4 /4 /2 |  |  |
| TOTAL |  | /60 |  |  |

| PLOT PLAN |  |  |
| --- | --- | --- |
| NAME: |  |  |
| Quality & Completeness of… |  |  |
| APPEARANCE | - House is ONLY solid line for ONLY exterior wall PERIMETER - Roof is ONLY hidden line - Only show YOUR property, plus closest utility locations - Do not show any offsets you used | /2 /2 /2 /2 |
| DIMENSIONS | - Property Line lengths - House exterior wall lengths - Closest distance from walls to closest property lines (perpendicular to all property lines - use PERP o-snap) - Distance from center of road to property line | /2 /4 /4 /2 |
| LABELING | - Square footage of house - Square footage of property - Address of property - Utilities (gas, water, sewer, phone, electricity) - North arrow | /2 /2 /2 /2 /2 |
| TOTAL |  | /30 |

| 3D VIEWS |  |  |  |
| --- | --- | --- | --- |
| NAME: |  |  |  |
| LEVEL 1: Kitchen Only |  |  |  |
| LEVEL 1 | APPEARANCE | - Visually appealing view - Matches Floor Plan | /10 /10 |
|  | DETAIL | - Counter thickness - Kick - Drawer & Cupboard fronts - Pull knobs/handles - Light switches & Wall sockets - Faucets - Light fixtures - Flooring pattern - Window /Door trim and visible depth (sill) - Appliance detail +2 BONUS: Extra details (plants, toaster, coffee maker, things not part of "the house") | /2 /2 /2 /2 /2 /2 /2 /2 /2 /2 /2 |
| TOTAL |  |  | /40 |
| LEVEL 2 & 3: Kitchen, & House |  |  |  |
| LEVEL 2 & 3 | KITCHEN | - Attention to Detail - Visually Appealing View | /5 /5 |
|  | HOUSE | - Attention to Detail - Visually Appealing View | /5 /5 |
| TOTAL |  |  | /20 |

| HOUSE MODEL |  |  |
| --- | --- | --- |
| NAME: |  |  |
| APPEARANCE | - Matches Floor Plan and Elevations |  |
| QUALITY | - Straight, smooth cuts on ALL foam edges - Square (90°) and vertical walls - Crisp, clean, glue-free ALL corners - Roof fits walls correctly - Consistent roof pitch | /3 /2 /5 /5 /5 |
| DETAIL | - Basic, Printed, Assembled (this is kind of a given) (+10) - Hand-made shingles added (+5) - Doors and windows made from card stock and glued on (+5) - See-through windows using a transparency for glass (+5) - Alternate materials used for finishes (+5) - Interior walls and details (+10) | /30 |
| TOTAL |  | /50 |

| SECTION VIEW (L2 & L3) |  |  |
| --- | --- | --- |
| NAME: |  |  |
| Quality & Completeness of… |  |  |
| DRAWING | - Roof Truss Detail - Studs - Floor Joists - Insulation - Foundation Walls - Footing Detail - Drain Tile & Drain Rock | /2 /2 /2 /2 /2 /2 /2 |
| ANNOTATING | - Roofing Details - Truss Details - Insulation Details (wall and ceiling) - Siding Details (including barriers) - Wall Details (including barriers) - Floor Joist Details - Foundation Details (including barriers) - Building Heights | /2 /2 /2 /2 /2 /2 /2 /2 |
| TOTAL |  | /30 |

| ANIMATED WALK-THROUGH |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NAME: |  |  |  |  |  |  |
| VIEW | A drunken three-legged Ox. | Careless showing of the house. Un-natural views, illogical camera placement. | Adequate showing of house. Main rooms shown,. | Good view of most of house, realistic offset, better than average "tour." |  | Excellent view of entire house; realistic offset; "natural" feel; I think I'm actually there. |
|  | 2 | 5 | 10 | 13 |  | 15 |
| MOVEMENT | A drunken three-legged Ox had a GoPro strapped to it. | Very rough, not much thought put into it, careless | Actually walks through the house, generally smooth, but you're glad you didn't pay much money for it. | Good view of most of house, realistic offset, better than average "tour," smooth. |  | Excellent view of entire house; realistic offset; "natural" feel; I think I'm actually there. |
|  | 2 | 5 | 10 | 13 |  | 15 |
| TOTAL |  |  |  |  | /30 |  |

| LEVEL 3 REQUIRED EXTRAS New for 21-22 |  |  |  |  |
| --- | --- | --- | --- | --- |
| NAME: |  |  |  |  |
| TOPOGRAPHY | What are some key features in your topography that you are most proud of? What was the most challenging part of topography for you? If you were to do it over again, what would you DO differently? |  |  |  |
|  | I cannot look away out of “sheer terror” 5 | It’s acceptable “It’ll work” 10 | Textbook awesomeness! “Hire me now!” 15 |  |
| COMPONENT | What component did you create for your house? What was the most challenging part of creating a component for you? What would you do differently? |  |  |  |
|  | I cannot look away out of “sheer terror” 5 | It’s acceptable “It’ll work” 10 | Textbook awesomeness! “Hire me now!” 15 |  |
| TOTAL |  |  |  | /30 |